DW1

Exercise 1

A beam simply supported at B and hinged at A supports a force F at D.

- Calculate the support reactions.

- Calculate the internal force vector in the straight sections G1 and G2.

Given: F = 5000 N

Exercise 2

A beam simply supported at B and hinged at A supports a force F at D and at H.

- Calculate the support reactions.

- Calculate the internal force vector in the straight sections: G1, G2, and G3.

- Deduce the type of loading in these sections.

Given: F = 5000 N

Exercise 3

A reinforced concrete beam is fixed at A, supports its own weight, and two forces F1 and F2 at B.

- Calculate the value of the distributed load q per linear meter due to its own weight.

- Calculate the support reactions.

- Calculate, as a function of x, the internal force vector in section G.

- Draw the diagrams of the non-zero components of the internal force vector along the beam.

Given:

F1=3000N, F2=2500N, volumetric weight of reinforced concrete = 25000 N/m3, section of the beam = 30×20 cm2