Chapter 4: Geometric Characteristics of Sections

- Static Moments

Let there be a flat surface S in the coordinate system (0,x, y) (Figure 1), considering an element dS relative to point M on this surface. The static moment AΔ​ of S relative to an axis Δ located at a distance δ from M is defined by:

AΔ=∫∫Sδ dS

If Δ coincides with one of the axes (0,x) or (0, y), we then obtain:

- The static moment of the surface S relative to the axis (0,x):

A0x=∫∫S y.dS

- The static moment of the surface S relative to the axis (0,y):

A0y=∫∫S x.dS

The unit of the static moment is obviously m3.

- Center of Gravity

The center of gravity or center of mass of surface S is the point G defined by its coordinates XG​ and YG​ such that:

XG=A0y/S= (∫∫S y.dS)/S                             YG= A0X/S= (∫∫S x.dS)/S

If a surface S is composed of n surfaces Si, then we have:

XG=1nXGi.SiS              and            YG=1nYGi.SiS

with:

- XGi ​​: x-coordinate of the center of gravity Gi ​ of the surface Si

- YGi​​: y-coordinate of the center of gravity Gi ​ of the surface Si

- Quadratic Moments

Let the flat surface S be in the coordinate system (0,x, y) as shown in Figure 1. The quadratic moment (or moment of inertia) IΔ​ of S relative to an axis Δ is defined by:

IΔ=∫∫Sδ2.dS

If Δ coincides with one of the axes (0,x) or (0, y), we then obtain:

- The quadratic moment of the surface S relative to the axis (0,x):

IOx=∫∫S y2.dS

- The quadratic moment of the surface SSS relative to the axis (0, y):

IOy=∫∫S x2.dS

The unit of the quadratic moment is obviously m4.

Huygens' Theorem

Let the axis Δ′ pass through the center of gravity G of the surface S. The quadratic moment IΔ​ is calculated from the quadratic moment IΔ′​ using the formula:

IΔ=IΔ′+S⋅d2

where d is the distance between the axis Δ and Δ′.

Applying Huygens' theorem to the axes (0,x') and (0, y') gives:

IOx​=IGx′ ​+ S⋅YG2

IOy=IGy′ + S⋅XG2​

If a surface S is composed of nnn surfaces Si​, we have:

IOxi=∑1nIOx       and      IOyi=∑1nIOy

with:

- IOxi: quadratic moment of surface Si​ relative to the axis(0,x)

- IOyi​​: quadratic moment of surface Si relative to the axis (0,y):

- Polar Quadratic Moment

The polar quadratic moment of a surface S relative to a point is defined by:

Ip=∫S r2.dS

After integration, the expressions for Ip​ for the most commonly used sections are found (see table alongside):

| Section | Ip​ |
| --- | --- |
| Solid Shaft | πD432​ |
| Hollow Shaft | π(D4-d2)32 |