Chapter 5: Pure Shear

- Definition

A beam or a section of a beam is said to be subjected to pure shear if the internal force tensor at each straight section can be summarized as:

τi=00Ty0Tz0

where:

- Ty ​: Shear force in the y-direction

- Tz​: Shear force in the z-direction

This is the general case; particular cases arise when one of the shear forces is zero.

- Shear Stress/Shear Force Relationship

The shear stresses distributed over the cross-section of the beam are statically equivalent to the shear force:

Ty = s⌡τ ds

Assuming that the shear stress is uniform across the entire cross-section, we have:

τ=Ty/S

​​

In the general case, we have:

τ(y)=Ty.A(y)b(y).IGZ

where:   A(y) = s1⌡y. ds    Static moment of section S1

- Ty​: Shear force

- b(y): Width of the beam at distance yyy from the neutral axis

- IGZ ​: Quadratic moment of the section

- Shear Deformation

The diagram resulting from the shear test is similar to that from the tensile test; it includes an elastic region and a plastic region.

The elastic region is governed by Hooke's law in shear:  τ=G⋅γ

Where:

- τ: shear stress

- G: shear modulus of the material

It is noted that G is related to E by the formula:     G=E/(2.(1+ν))

Where:  ν is Poisson's ratio.

- Design Criteria

The main criterion used to calculate the transverse dimensions of a beam subjected to pure shear is the stress criterion, which requires that the material remains (with a safety factor), within the elastic region:

τmax≤ τe/s

Where:

- τmax​: maximum applied stress

- τe​: shear yield strength of the material

- s: safety factor (≥ 1)