DW5

Exercise 1

A beam with a tubular section of length L is fixed at one end and subjected to a torsional moment Mt at the other end.

- Calculate the stress on the inner surface of the beam

- Calculate the stress on the outer surface of the beam

- Plot the stress distribution on the cross-section

- Check the strength of the beam

- Calculate the angle of rotation of a section located at the midpoint of the beam

- Calculate the angle of rotation of the section subjected to the torsional moment

Given:
Mt=200 Nm.M, Dext=40 mm, Dint​=32mm, L=0.5 m, G=78GPa, τe=60 MPa, s=2

Exercise 2

A solid cylindrical shaft, with length LLL and diameter DDD, must transmit a maximum power of P=300 CH at a speed of 3000 rpm.

- Determine the torsional moment Mt transmitted.

- Determine the minimum diameter the shaft must have to safely transmit this torsional moment.

Given:
L=3 m, τe=25×107 Pa, safety factor s=5, 1 CH=735.49 W

Exercise 3

A ship shaft 30 meters long must transmit a torsional moment of Mt=1.9 MNm. It is made from steel with a shear modulus G=82,000N/mm2 and a shear yield strength τe=50 N/mm2.

Calculate and compare its outer diameter, weight, and rotation between its two ends for two values of K= Dint/Dext

- K1=0  for a solid shaft

- K2​=0.9 for a hollow shaft

Given:

- Steel density ρs=7800 kg/m3

- Safety factor s=1