DW6

Exercise 1

The beam AB with a rectangular cross-section is simply supported at both ends and is subjected to a uniformly distributed load q along its entire length L.

- Plot the diagrams of the non-zero components of the internal force system (torseur of cohesion) along the beam. From this, deduce the most critical sections.

- Calculate the maximum normal stress and verify the beam's strength.

Given:   σe=160 N/mm2

Exercise 2

Redo the questions from Exercise 1 for the following beam

Exercise 3

The beam AB is fixed at point A and subjected to a concentrated force F at point B.

- Plot the diagrams of the non-zero components of the internal force system (torseur of cohesion) along the beam.

- Determine the expression for the deflection v(x) (equation of the deformation) and deduce the maximum deflection.

- Check for the three given sections whether the deflection exceeds the limit value: vlim=1 mm with a safety factor s′=2.

Given: E=200,000 MPa