### Chapter 6: Torsion

#### 1. Definition

A beam or a segment of a beam is said to be subjected to simple torsion if the internal force system can be reduced to:

τi=0Mt0000  Where Mt​ is the torsion moment around the x-axis.

Note: In this course, we only consider the torsion of beams with circular cross-sections.

#### 2. Stress/Strain Relation

Consider a beam with a circular cross-section fixed at one end and subjected to a torsion moment Mt ​at the other end. It is experimentally observed that each cross-section of the beam rotates by an angle proportional to its abscissa. There is no longitudinal deformation, so there is no normal stress; only tangential stresses exist.

The angle of deformation γ is called the distortion. Since γ is small, we can approximate γ by tan(γ), and we obtain:

γ = r .(dα/dx)= r.θ​

Where:

- γ is the angular distortion,

- r is the radius of the section,

- θ is the rotation of the section,

- dx is the infinitesimal length.

The diagram resulting from the torsion test is similar to that from a tensile test, and it has an elastic domain and a plastic domain.

The elastic domain is governed by the shear Hooke's law:

τ=G.γ

τ=G.r. θ

Where:

- τ is the shear stress,

- G is the shear modulus of the material.

We note that G is related to the Young's modulus E by the formula:

G=E/2(1+ν)

Where ν is Poisson's ratio.

The formula τ=G.r.θ​ also gives the distribution of shear stress .

over the beam's cross-section.

The shear stress is zero at the center and maximum at the outer surface.

#### 3. Stress/Moment Relation

Consider a surface element ds of a beam's cross-section subjected to a torsion moment. 	This surface element, located at a distance r from the center of the section, is subjected to the shear stress τ.

The torsion moment relative to this surface element ds is:

dMt = τ.ds.r

Since τ=G.r.θ ​, we obtain:

dMt=G.r2.θ.ds

The total moment is obtained by integrating dMt

over the surface S:

Mt=∫SG.r2.θ.ds = G. θ ∫S r2.ds = τr ∫S r2.ds;  then Mt = τr . Ip

The term ∫S r2.ds is the polar moment of inertia Ip​

of the section and can be easily calculated.

The maximum shear stress occurs at r=R, and is given by:

τmax = Mt.R / Ip

#### 4. Relation between Rotation Angle and Moment

To calculate the rotation angle of a cross-section at abscissa x:

θ = dα /dx = Mt / (G.​Ip)​ then α = ∫x0 (Mt / (G.​Ip))​ dx

Thus, the rotation angle between two extreme sections of a beam of length L is:

α=∫L0 (Mt / (G.​Ip))​ dx = Mt .L / (G.​Ip)     Where:

- Ip​ is the polar moment of inertia of the cross-section,

- G is the shear modulus,

- Mt​ is the torsion moment.

#### 5. Design Criteria

As with tension, two criteria can be used to calculate the transverse dimensions of a beam subjected to simple torsion:

- The stress criterion and

- The rotation criterion.

The stress criterion imposes that the material stays within the elastic domain (with a safety factor):

τmax ≤ τe/s     Where:

- τmax​ is the maximum applied shear stress,

- τe​ is the shear yield strength of the material,

- s is the safety factor (s≥1s \geq 1s≥1).

The rotation criterion imposes that the angle between the extreme sections does not exceed a given limit.

#### 6. Power Transmission

In mechanics, a shaft rotating at an angular speed ω (rad/s) and subjected to a torsion moment Mt​ transmits power P such that:

P = Mt . ω = 2 π f Mt

Where f is the rotational speed (in revolutions per second).