CENTRE UNIVERSITAIRE  DE MILA

Département Génie civil et Hydraulique

Course : Dynamique Des Structures

TD : SERIES OF EXERCISES No. 02

(Undamped and damped free vibrations)

Exercise N° 01 :

K

C

M

Write the equation of motion for the single degree of freedom system shown in Figure 1.1 with respect to the position of static equilibrium.

x(t)

P(t)

Exercice N° 02 :

Fig. 1-1

Considering an undamped vibration system (C = 0), (see Fig. 1-2).

With. M=8,75*105 kg , K=3,5*106 N/m and the initial conditions:

C

K

x(t)

M

Fig. 1-2

X(t=0) = 4,5 cm , X(t=1,2 sec) = 4,5 cm

Determine:

- a) The displacement at time t = 2.4 sec ?

- b) The amplitude of the free oscillations ?

Exercise N° 03 :

Let's assume for the structure given by (Fig. 1-3), the mass and stiffness have the following values:

C

K

x(t)

M

Fig. 1-3

M = 3,5 * 105 kg, K = 7*106 N/m

If the system is set into free vibration with the initial conditions

X(0) = 1,75 cm et X’(0)=14 cm/sec.

Determine the displacement and velocity at time t = 1 sec assuming that:

- C = 0 (undamped system) ?

- C = 4,9 *105 N.sec/m ?

x(t)

C

K/2

K/2

M

Fig. 1-4

Exercice N° 04 :

Consider the system given by (fig. 1-4) avec M=2000kg, K=300 kN/m et  = 5% ,

Determine:

a) The angular frequency?

b) The damping coefficient C?

c) The logarithmic decrement  ?

d) The number of cycles necessary and the time for the amplitude to be reduced from its initial value of 1 cm to 0.1 cm?

Exercise N° 05 :

The weight p of a structure is 1000 kN. This structure is released at time (t = 0 sec) from an initial displacement of 3 cm.

If the maximum displacement after one cycle is 2.2 cm at time t = 0.64 sec.

Determine:

x(t)

C

K/2

K/2

P

Fig. 1-5

- The damping factor  ?

- The spring stiffness K ?

- The damping coefficient C ?

------- End of the series. -------