MILA UNIVERSITY CENTER

Department of Civil Engineering and Hydraulics

Course : Structural Dynamics

TD : SERIES OF EXERCISES No. 03

(Response of an S.D.O.F. system to an harmic load)

Exercice N° 01 : (Cutch‐up exam 2023)

M

3m

EI

EI

EI

1/ The system is set into free oscillation (X0=3cm) and the time is measured for it to pass six (6) times through the same point (t=2.4 s) where the displacement is reduced to 0.2 cm

a) Determine the natural frequency of the system

b) Determine the damping factor ξ

c) Determine the stiffness K of the system

2/ By placing on the platform an electric motor that exerts on it a horizontal force:

F(t) = 250 sin (10t) (KN)

If the mass of the motor is neglected, determine:

a) The maximum response of the platform

b) The maximum stress in each pole, knowing that the cross-sectional shape is a 30 cm square

Exercice N° 02 : (Cutch‐up exam 2020)

Consider the structure represented by the figure below: M= 1000kg and E= 2×104 MPa

The stiffness of a column is K=3EI/h3

a) By recording the free vibration response of this structure, it is observed that the ratio between two successive peaks is 1.37.

- Determine the damping factor ξ of this structure?

b) For the rest of the exercise, consider that the base of this structure is subjected to an earthquake that can be assimilated to a harmonic excitation of the form Xg(t) = 5sin30t  (cm)

- Determine the dimension a of the pole's cross-section necessary to introduce resonance in the structure.

x(t)

C

K/2

K/2

M

h=3 m

a

a

L=3 m

Exercise N° 03 :

F(t)

F(t)

M=7500kg

M=8300kg

K/2

K/2

A rotating machine, supported by the structure shown in the figure below, generates a horizontal harmonic force with an amplitude of 550 N at a frequency of 835 Hz in the plane of the structure. The mass of the machine m = 7500 Kg is added to the mass of the structure M= 8300kg

Question:

- Determine the steady-state amplitude if ξ is equal to 5%

- Determine the steady-state amplitude if the frequency of the external

force is equal to the natural frequency of the structure.

With: K = 6.5 106 N/m

------- Fin de la série. -------