MILA UNIVERSITY CENTER

Department of Civil Engineering and Hydraulics

Course : Structural Dynamics

TD : SERIES OF EXERCISES No. 04

(Response of an S.D.O.F. system to an impulsive load)

Exercice N° 01 : The system below is subjected to an excitation P(t) shown on the graph. The initial conditions are:

P(t)

t(s)

P0 =1KN

4

2

0

P(t)

x(t)

X0 = 2cm and initial velocity X0=1cm/s

Questions : Determine x(t=1s) ; x(t=3s) ; x(t=5s)

Given: M= 1000Kg , K=1KN /cm et C= 0

Exercise N° 02 :

Consider a system characterized by a mass M = 100 kg and a spring with stiffness K = 10000 N/m.

This system is excited by a rectangular load P0=100 N for a duration t1.

P(t)

t

P0

t1

Determine the time at which the displacement is maximum and calculate this displacement considering the following cases.

- t1 = T/8

- t1 = T/2

- t1 = 5T/4

Where T is the natural period of the structure under free vibration.

Exercice N° 03 :

Consider a system characterized by a mass M = 1000 kg and a spring with stiffness K = 10000 kN/m.

This system is excited by a load represented in the following figure with t1 = 0.05 sec and P0 = 100 N

- Determine the time tm at which the system reaches its maximum.

P(t)

t

P0

t1=0.05

Calculate this displacement.

Exercice N° 04:

Consider the structure represented in the following figure with M = 1000 kg and K = 10000 kN/m and with h=3m, L=3m ;

The section of the column is : 20*20 cm2. This structure is excited by an explosion modeled as a triangular load.

Determine the maximum shear force in the columns and the maximum normal stress in the columns.

E = 2*105 MPa. (Neglect the normal force in the columns).

P(t)

t

P0 = 1000 N

t1 = 0.05 sec

P(t)

h=3 m

L=3 m

------- end of serie. -------