LW3 - LRC SERIES FREE OSCILLATIONS

I. OBJECTIVES:

- Show that an LRC circuit can be an oscillation seat.

- Show that the capacitor (C) and inductor (L) are causing oscillations in an LRC circuit.

II. THEORY:

II. 1. Equations:

We consider the LRC circuit in series schematized in opposite figure 1. The Low Frequency Generator (LFG) produces a crenellated voltage equal to E. If we take the signal positive half-period, we get:

Figure II. 1: Serial LRC Circuit

uL+uR+uC=E  uR= R·iuL=L∙didt+r∙i    →  R+r∙i+L∙didt+uC= E

i=dqdt    →R+r∙dqdt+L∙d2qdt2+uC=E

q=C∙uc  → R+r∙CduCdt+L∙C∙d2uCdt2+uC=E

d2ucdt2+R+rLducdt+1L∙Cuc=1L∙CE 2λ= R+rL  ,   ω02= 1L∙C    →  uc+2λ∙uc+ω02∙uc=ω02∙E

Finally, we get the equation in the following form:

uc+2λ∙uc+ω02∙(uc-E)=0

If we set UC = uc - E, this last equation will take the following form:

UC+2λ∙UC+ω02∙UC=0

Which represents the free movement of the system. The quantity ω0 is called the resonant frequency and λ is the damping factor.

For fixed L and C values, the term [R+rLdUCdt=2λ∙Uc=2λ∙uc] determines the regime type (periodic, pseudo-periodic, aperiodic, critical).

II. 2. Different regimes of the evolution of the voltage uc(t):

II. 2. 1. Periodic Regime (Sinusoidal):

For very low or zero values of the total resistance Rtot, the voltage uC(t) across the capacitor has undamped free oscillations, this is the periodic regime. In this case, the voltage uC(t) periodically passes through zero values. The time between two successive passes through a zero value, with a slope of the same sign, is the eigenperiod T0 of the undamped free oscillations. The amplitude of the oscillations remains constant over time (Figure 2-a).

UCt=uCt-E=Umcosω0t+φ =Umcos2πT0t+φ

The magnitude φ is called the phase at the origin of dates and is expressed in radians.

II. 2. 2. Pseudo-periodic regime:

For low values of the total resistance Rtot, the voltage uC(t) across the capacitor has damped oscillations, this is the pseudo-periodic regime. In this case, the voltage uC(t) periodically passes through zero values. The time between two successive passes through a zero value, with a slope of the same sign, is the pseudo-period T of the damped oscillations. In this case, the amplitude of the oscillations decreases over time (Figure 2-b).

UCt=uCt-E=Um∙e-λtcosωt+φ=Um∙e-λtcos2πTωt+φ

Um   is the amplitude of the oscillations and ω is the angular frequency given by: ω=ω02-λ2=2πT

II. 2. 3. Critical Regime:

For a particular value of  RTot=RC=2L/C   , The regime is referred to as a critical aperiodic regime. In this case, the voltage UC(t) across the capacitor tends more rapidly to zero. The critical regime is difficult to determine experimentally (Figure 2-c).

II. 2. 4. Aperiodic Regime

For very large values of Rtot, the voltage uC(t) dampens very quickly, this is the aperiodic regime. The discharge is observed without the voltage uC(t) oscillating (Figure 2-c).

Figure II. 2: Different oscillation regimes

III. HOME WORKS:

Complete the following tables:

III. 1. The influence of C on voltage uC (t):

| R=10 Ω; L=1 mH=0.001 H, r=1.3 Ω |  |  |  |  |
| --- | --- | --- | --- | --- |
| 40 | 30 | 20 | 10 | C (nF) |
|  |  |  |  | C (F) |
|  |  |  |  | ω0(rd/s)=1/LH∙CF |
|  |  |  |  | λ(Ω/H) = (R + r)/2L |
|  |  |  |  | ω(rd/s) =ω02-λ2 |
|  |  |  |  | Tcal(s) =2π/ω(rd/s) |
|  |  |  |  | Tcal (µs) |

III. 2. The influence of L on voltage uC (t) :

| R=10 Ω , C= 20 nF=20×10-9F |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 | L(mH) |
|  |  |  |  | L(H) |
|  |  |  |  | ω0(rd/s)=1/LH∙CF |
|  | 3.5 | 1.9 | 1.3 | r(Ω) |
|  |  |  |  | λ (Ω/H) = (R + r)/2L |
|  |  |  |  | ω(rd/s) =ω02-λ2 |
|  |  |  |  | Tcal(s) =2π/ω(rd/s) |
|  |  |  |  | Tcal (µs) |

III. 3. The influence of R on voltage uC (t) :

| L= 1 mH=0.001 H, r=1.3 Ω , C=20 nF=20×10-9 nF |  |  |  |  |
| --- | --- | --- | --- | --- |
| 40 | 30 | 20 | 10 | R(Ω) |
|  |  |  |  | ω0(rd/s)=1/LH∙CF |
|  |  |  |  | λ (Ω/H) = (R + r)/2L |
|  |  |  |  | ω(rd/s) =ω02-λ2 |
|  |  |  |  | Tcal(s) =2π/ω(rd/s) |
|  |  |  |  | Tcal (µs) |

| Full name | Group | Assiduity (02pts) | Report (18pts) | Grades / 20 |
| --- | --- | --- | --- | --- |
| Date: …………………………………… ……………….…Time: ……………………………Lab No.……………… |  |  |  |  |

IV. PROCEDURES AND DATA ANALYSIS:

- The serial LRC circuit diagrammed above (Figure 1) is constructed and the necessary connections are made to visualize the voltage uC (t) across the capacitor on the oscilloscope channel 1.

- The inductor has an adjustable inductance L and an internal resistance r.

- The function generator is set to produce a peak-to-peak voltage of VPP = 4V and frequency = 1 KHz.

IV. 1. The influence of C on voltage uC (t):

1. Complete the following table:

| R=10 Ω; L=1 mH, r=1.3 Ω |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0.040 | 0.030 | 0.020 | 0.010 | C (µF) |
|  |  |  |  | C(μF) |
|  |  |  |  | Tmeas (µs) |

2. Plot the graph Tmeas=f(C)

Figure IV. 1: …………………………………………………………………………

3. Determine graphically the slope (SC) of the graph Tmeas=fC by specifying its SI unit?

IV. 2. The influence of L on voltage uC (t) :

1. Complete the following table:

|  | R=10 Ω , C= 0.020 µF |  |  |  |
| --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 | L(mH) |
|  |  |  |  | LmH |
|  |  |  |  | Tmeas (µs) |

2. Plot the graph Tmeas=f(L)

Figure IV. 2: …………………………………………………………………………

3. Determine graphically the slope (SL) of the graph Tmeas=fL by specifying its SI unit?

IV. 3. Determination of the pseudo-period expression:

1. Deducing from questions IV. 1 and IV. 2 above that the pseudo-period can be written in the following form T =k L·C  where k is a constant.

2. By the dimensional analysis, verify that the constant k has no dimension and is simply expressed as a function of the number π.

IV. 4. The influence of R on voltage uC (t) :

1. Complete the following table:

| L= 1 mH, r=1.3 Ω , C=0.020 µF |  |  |  |  |
| --- | --- | --- | --- | --- |
| 30 | 20 | 10 | 0 | R(Ω) |
|  |  |  |  | Tmeas (µs) |

2. For a very large resistance R, are there oscillations, justify your answer?

3. Predict what would happen if RT = 0 and how would call this type of circuit?

V. Recaps :

Indicate by YES or NO in the corresponding box about the influence of the different parameters?

| Parameters | C | L | R |
| --- | --- | --- | --- |
| Period |  |  |  |
| Amplitude |  |  |  |

VI. CONCLUSION:

VII. BIBLIOGRAPHY:

- https://www.subr.edu/assets/subr/ElectricalEngineering/TargetedInfusion/Files/ENGR-120_RLC-Circuits.pdf

- https://photonics101.com/basic-ac-circuits/serial-rlc-circuit

- https://www.owlnet.rice.edu/~phys102/Lab/RLC_circuits.pdf

- https://spinningnumbers.org/a/rlc-natural-response-derivation.html

- http://mokrini.e-monsite.com/medias/files/ph-tp8-rlc.pdf

- http://www.sciences.univ-nantes.fr/physique/perso/gtulloue/Elec/Transitoire/Condensateur1_flash.htm

- http://mokrini.e-monsite.com/medias/files/ts-phy-chap8-oscillations-libre-dans-le-dipole-rl.pdf

- http://www.physagreg.fr/CoursTS/Physique/Cours/Physique-C-chap8-oscillations-electriques.ppt

- http://montblancsciences.free.fr/terms/physique/cours/p10.htm

- http://e.guimberteau.free.fr/Site/TS/Physique/C3%20cours.htm

- https://docplayer.fr/22000964-Tp-8-oscillations-dans-un-circuit-rlc-correction.html