### Problem 1

The mass percent composition of the chemical compounds in a sample is as follows:

CaO = 62.8; SiO₂ = 24.4; Al₂O₃ = 4.8; Fe₂O₃ = 3.2; SO₃ = 2.4; MgO = 0.8; Loss = 1.5 (%).

- What type of this sample is it? Justify your answer.

- What is the name of the compound responsible for the initial setting of cement? What is its simplified formula and its chemical formula?

- Calculate the percentage of the compound responsible for resistance to seawater.

- Calculate the percentage of the compound responsible for the final setting of cement.

Given: The molar masses are as follows:

Ca = 40.08 g/mol; Al = 26.98 g/mol; O = 16 g/mol; Fe = 55.85 g/mol; Si = 28.09 g/mol

Corrosion

- Give the main factors influencing corrosion.

- Give three types of corrosion.

- An iron plate is immersed in an acidic solution containing permanganate ions MnO4−. After a certain period of time, Fe2+ ions are detected in the solution and the purple color of permanganate gradually disappears. The Standard reduction potentials at 25 °C versus the Standard Hydrogen Electrode are:

Fe2+ /Fe E° = − 0.44 V
MnO4− / Mn2+  E° = +1.51 V

- Identify the redox couples involved in the corrosion process.

- Write the oxidation half reaction of iron.

- Write the reduction half reaction of permanganate in acidic medium.

- Write the overall redox reaction.

- Conclude whether the corrosion of iron is thermodynamically possible in this solution.

If this metal sample is a cylindrical with a diameter of 5 cm and a height of 1.5 cm is immersed for 24 hours and the mass loss is 0.48117 mg.

- Calculate the corrosion rate of this metal.