Abdelhafid BOUSSOUF University of Mila

Faculty Of Science & Technology

Department of Process Engineering

Homogeneous reactor

Second semester exam

2025/2026

Activity 1 (4 points)

Are the following sentences true or false, if they are false, give the correct sentence:

1/ According to the degree of mixing, reactors can be classified as: Batch, CSTR, and PFR.

2/ According to the degree of mixing, reactors can be classified as: Closed and Open.

3/ In the most cases, the performances of PFR are better than those of CSTR (for same conditions).

4/ PFR don’t required a hight flow (input and output) control.

Activity 2 (7 points)

Part 1:

We examine the response: A+B => 2C+D

This reaction (of the second order, first order with regard to A and B) occurs in liquid phase where two moles/l of A and 2mol/l of B are added to the reactor's feed. Our goal is to get a C concentration of 3.6 moles/l.

- Provide the speed equation as a function of temperature T and conversion rate XA.

- Assume that the reaction occurs in a 5m3 closed reactor with a rate constant of k=2.8 litre/mol.h. in an isothermal environment (T=20°c). Determine how long it will take to get the required C concentration.

- Let's say we have a 3m3 open reactor. Determine the XA conversion rate.

Part 2:

We consider a series of two open reactors of volume 2m3 and 4m3.

1/ Determine the value of the conversion rate XA at the output constitute by:

- 2m3 reactor followed by a 4m3 reactor;

- 4m3 reactor followed by a 2m3 reactor;

2/ Compare the performances of various reaction systems.

Activity 3 (6 points)

Consider the chemical reaction: A Products with first order kinetics.

Knowing that: k=0.1s-1, the flow rate F= 50 l/s, and the reaction volume V=100 l;

- Calculate the conversion at the outlet of each of the following reactors:

a/ CSTR

b/ PFR

- Give a conclusion (by comparing the two reactors).

Activity 4 (3 points)

- The combustion equation for ethanol is: C2H5OH + 3O2 → 2CO2 + 3H2O

3 moles of ethanol and 5 moles of dioxygen are reacted together; one of the reagents disappears completely. How many moles of carbon dioxide, CO2, are formed at the end of the reaction?

Good Luck

جامعة عبد الحفيظ بالصوف – ميلة

كلية العلوم و التكنولوجيا

قسم هندسة الطرائق

مقياس المفاعلات المتجانسة

إمتحان السداسي الثاني

2026/2025

التمرين 1 (4 نقاط)

هل الجمل التالية صحيحة أم خاطئة؟ إذا كانت خاطئة، فاكتب الجملة الصحيحة:

/1تُصنف المفاعلات، وفقًا لدرجة الخلط، إلى: مفاعلات الدفعات، ومفاعلات الخلط المستمر (CSTR)، ومفاعلات التدفق الأنبوبي (PFR).

/ 2تُصنف المفاعلات، وفقًا لدرجة الخلط، إلى: مفاعلات مغلقة، ومفاعلات مفتوحة.

/ 3في معظم الحالات، و في ظل نفس الظروف، يكون أداء مفاعلات التدفق الأنبوبي (PFR) أفضل من أداء مفاعلات الخلط المستمر (CSTR)

/4  لا تتطلب مفاعلات التدفق الأنبوبي (PFR) تحكمًا عاليًا في التدفق (المدخل والمخرج).

التمرين 2 (7 نقاط)

لنفترض التفاعل التالي: A+B  2C+D

الجزء الأول:

يحدث هذا التفاعل من الدرجة الثانية، )الأولى بالنسبة لـ A وB في الطور السائل عند الضغط الجوي(. تتم تغذية المفاعل بإضافة 2 مول/لتر من A و2 مول/لتر من .B نريد الحصول على تركيز C يساوي 3.6 مول/لتر.

1/ أعط عبارة السرعة بدلالة معدل التحويل XA ودرجة الحرارة T.

2/ لنفترض أن التفاعل يحدث في مفاعل مغلق سعته 5م3 في بيئة متساوية الحرارة (T=20°c) مع ثابت سرعة k=2.8لتر/مول. احسب الوقت اللازم للحصول على التركيز المطلوب للعنصر C.

3/ إذا كان لدينا مفاعل مفتوح بحجم 3 م3، احسب معدل التحويل XA.

الجزء الثاني:

ندرس سلسلة من مفاعلين مفتوحين متتاليين بحجم 2 م3 و 4 م3

1/ حدد قيمة معدل التحويل XA عند المخرج، والمكون من:

- مفاعل 2 م3 يليه مفاعل 4 م3

- مفاعل 4 م3 يليه مفاعل 2 م3

2/ قارن بين أداءات مختلف الأنظمة.

التمرين 3 (6 نقاط)

لنفترض التفاعل الكيميائي التالي: A      النواتج، والذي يخضع لحركية من الرتبة الأولى.

مع العلم أن: k=0.1s-1، ومعدل التدفق لتر/ثانية F = 50، وحجم التفاعل لتر V = 100 ؛

- احسب معدل التحويل عند مخرج كل من المفاعلين التاليين:

أ/ مفاعل الخلط المستمر (CSTR)

ب/ مفاعل التدفق الأنبوبي (PFR)

- اعط خلاصة عامة (بمقارنة المفاعلين).

التمرين 4 ( 3نقاط)

- معادلة احتراق الإيثانول هي: C2H5OH + 3O2 → 2CO2 + 3H2O

تتفاعل 3 مولات من الإيثانول و5 مولات من ثنائي الأكسجين معًا؛ ويختفي أحد المتفاعلات تمامًا. كم مولًا من ثاني أكسيد الكربون (CO2) يتكون في نهاية التفاعل؟                                                                                                         بالتوفيق