# Chapter 8: Using MATLAB Toolboxes

MATLAB is not only a powerful numerical computing environment but also a flexible platform that can be extended through toolboxes. A toolbox in MATLAB is a collection of specialized functions (M-files) designed to perform specific tasks in various engineering and scientific fields. Each toolbox focuses on a particular area such as signal processing, control systems, optimization, image analysis, or machine learning.

Toolboxes are extremely useful for engineers because they save development time and provide reliable, pre-tested algorithms that can be easily integrated into MATLAB scripts, functions, or Simulink models.
In mechanical engineering, toolboxes are widely used for vibration analysis, dynamic modeling, system control, optimization of parameters, and simulation.

## 8.1 Installing and Managing Toolboxes

MATLAB allows users to install and manage toolboxes through several methods:

### a) Using the Add-On Explorer

- Go to the Home tab on MATLAB’s toolbar.

- Click Add-Ons → Get Add-Ons.

- Search for the desired toolbox (e.g., Control System Toolbox).

- Click Install and MATLAB will automatically download and install it.

### b) Using the Command Window

You can check the installed toolboxes using:

ver

To see a list of all available functions in a toolbox:

help toolboxname

Example:

help signal

If a toolbox is missing, MATLAB will show an error message such as:

Undefined function or variable 'tf'.

which means that the Control System Toolbox is not installed.

## 8.2 Common MATLAB Toolboxes

Below are the most frequently used toolboxes in mechanical and scientific applications.

### 8.2.1 Signal Processing Toolbox

This toolbox provides tools for analyzing, filtering, and visualizing signals.

Key Functions:

fft(x)        % Fast Fourier Transform

ifft(X)       % Inverse FFT

filter(b,a,x) % Applies a digital filter

spectrogram(x)% Computes the signal spectrum

Example:

t = 0:0.001:1;

x = sin(2*pi*50*t) + sin(2*pi*120*t);

Y = fft(x);

plot(abs(Y));

title('Signal Spectrum');

Application in Mechanics:
Used to analyze vibration data from accelerometers or rotating machinery to detect unbalance or bearing faults.

### 8.2.2 Image Processing Toolbox

This toolbox provides algorithms for image analysis, enhancement, and visualization.

Key Functions:

imread('image.jpg')

imshow(I)

imadjust(I)

edge(I, 'canny')

Example:

I = imread('metal_surface.jpg');

BW = edge(rgb2gray(I),'canny');

imshow(BW);

title('Detected Edges of Metal Surface');

Application in Mechanics:
Used for defect detection on surfaces, material inspection, or automated visual measurements.

### 8.2.3 Control System Toolbox

Essential for modeling, analyzing, and designing control systems.

Key Functions:

tf(num,den)      % Transfer function

step(sys)        % Step response

bode(sys)        % Frequency response

feedback(sys1,sys2)

Example:

num = [1];

den = [1 3 2];

sys = tf(num, den);

step(sys);

title('Step Response of Control System');

Application in Mechanics:
Used for the design and tuning of PID controllers in mechatronic systems.

### 8.2.4 Optimization Toolbox

Used to minimize or maximize objective functions under constraints.

Key Functions:

fminbnd(fun,a,b)

fminsearch(fun,x0)

fmincon(fun,x0,A,b)

Example:

fun = @(x) (x-3).^2 + 2;

x = fminbnd(fun, -5, 5)

Application in Mechanics:
Optimization of design parameters, such as minimizing stress or energy consumption.

### 8.2.5 Statistics and Machine Learning Toolbox

Provides tools for data analysis, regression, and predictive modeling.

Key Functions:

mean(x)

std(x)

fitlm(X,Y)

kmeans(X,k)

Example:

load fisheriris

X = meas(:,1:2);

[idx,C] = kmeans(X,3);

gscatter(X(:,1),X(:,2),idx);

Application in Mechanics:
Used for fault classification, predictive maintenance, and data-driven decision-making.

### 8.2.6 Simulink

Simulink is an interactive graphical environment for modeling, simulating, and analyzing dynamic systems.

Example:
To open Simulink:

simulink

Application in Mechanics:
Used to simulate mechanical systems, control loops, and multi-domain physical systems such as hydraulic or thermal models.

## 8.3 Remarks and Best Practices

- Always check toolbox licenses before use with:

- license('test','toolbox_name')

- Keep MATLAB and all toolboxes updated for maximum compatibility.

- Use which functionname to locate which toolbox a function belongs to.

- Organize your code to avoid dependency errors when sharing files.

## 8.4 Exercises

### Exercise 1:

Use the Signal Processing Toolbox to analyze the vibration signal:

t = 0:0.001:1;

x = sin(2*pi*60*t) + 0.5*sin(2*pi*120*t);

Tasks:

- Compute and plot its frequency spectrum using fft.

- Identify the main frequency components.

### Exercise 2:

Using the Control System Toolbox, create a transfer function:
[
H(s) = \frac{10}{s^2 + 3s + 10}
]

- Plot the step response.

- Determine the steady-state value.

### Exercise 3:

Using the Optimization Toolbox, minimize the function:
[
f(x) = x^2 + 4x + 4
]
using fminbnd.

## 8.5 Solutions

### Solution 1:

Y = fft(x);

f = (0:length(Y)-1);

plot(f, abs(Y));

title('Frequency Spectrum');

### Solution 2:

num = [10];

den = [1 3 10];

sys = tf(num, den);

step(sys);

### Solution 3:

fun = @(x) x.^2 + 4*x + 4;

xmin = fminbnd(fun, -10, 10)